L20/540 - SLIDING FOLDING PARTITIONS TO …………………………………..

Refer to drawings: ………………………………………………………………………

Manufacturer and reference:  100 Series Operable Wall by Building Additions Ltd

Unit C1-C2, Southgate

Commerce Park

Frome

BA11 2RY

T: 01373 454577

F: 01373 454578

www.buildingadditions.co.uk

Characteristics:	Manually operated modular steel framed panel system with internal winding scissor action locking mechanism (quick action turn-cam mechanisms are not acceptable). Top and bottom retractable seals and internal telescopic end closure panel (seal pressure must be at least 2KN per linear metre to maintain acoustic integrity).

Colour/Finishes:

Furniture:

Pass Door:

Semi-automatic Operation:

Track and Roller System:	, heavy duty extruded aluminium top track with integral ceiling trim, powder coat finish white. No floor tracks.

Acoustic Rating: 	Rw  in accordance with ISO 140 Part 3. Current and correct certification must be produced. Building Additions will conduct an on site acoustic test after installation if so required.